Data Logging System:

The application can be used as a data logging system for embedded devices or industrial automation systems. The ADC thread can read sensor data (e.g., temperature, pressure, voltage), the RTC thread can provide accurate timestamps, and the EEPROM thread can store the sensor readings along with their timestamps in the EEPROM for non-volatile data storage. This data can be later retrieved and analyzed for monitoring, troubleshooting, or process optimization purposes.

U-boot i2c command:

$ i2c dev 1

$ i2c probe

$ i2c md

$ i2c mm/mw

ADC configuration in U-Boot:

- Enabling Clock for TSC_ADC_SS

clock enable register base address:

| CM_WKUP | 0x44E0_0400 | 0x44E0_04FF | 256 Bytes | Clock Module Wakeup Registers |
| --- | --- | --- | --- | --- |

ADC clock offset address:

BCh CM_WKUP_ADC_TSC_CLKCTRL 							Section 8.1.12.2.48

Uboot file path:

\arch\arm\mach-omap2\am33xx\clock_am33xx.c    -- &cmwkup->wkup_adctscctrl,

\arch\arm\include\asm\arch-am33xx\cpu.h      // all the clock CM_WKUP Registers are defined here (ex: wkup_adctscctrl for adc)

MUX configuration for I2c1 device controller.  (board/ti/am335x/mux.c)

To read adc register data bellow commands:

$ md <address (base+offset)> <count>

$ mm [<address>]

Accelerometer (ADXL345) EEPROM (AT24C02) Test in U-Boot:

$ Rohith=> i2c mw 0x50 0x00 (0x01 data) (1 len) //write command byte byte write. (mm will not work for eeprom auto increment of data)

$ Rohith=> i2c md 0x50 00 10 (display the data of 10 bytes length).

Same way we can access the register of ADXL345.

To read x,y,z reading from uboot we need to enable power sequencing.

I2c command can be initiated by setting the measure bit (Bit D3) in the POWER_CTL register (Address 0x2D).)

Measure Bit

A setting of 0 in the measure bit places the part into standby mode, and a setting of 1 places the part into measurement mode. The ADXL345 powers up in standby mode with minimum power consumption.

&sudo apt install libiio-utils

km@KM-BBB:~$ iio_

iio_adi_xflow_check  iio_info             iio_writedev

iio_attr             iio_readdev

iio_genxml           iio_reg

Kernel Space:

Device tree initialization for tsc ADC:

\Documentation\devicetree\bindings\input\touchscreen

arch/arm/boot/dts/am33xx.dtsi

/*AM33XX_PADCONF(AM335X_PIN_GPMC_A0, PIN_INPUT_PULLUP | SLEWCTRL_SLOW, MUX_MODE7) */

The reason we can't use the device files from the /sys/bus/iio/devices/ or /dev/input/devices/ directories is that they don't provide direct access to the accelerometer registers. These directories provide a higher-level interface to the sensor data, abstracting away the low-level register access.

- /sys/bus/iio/devices/: This directory is part of the Industrial I/O (IIO) subsystem in the Linux kernel, which provides a unified interface for various sensors and industrial I/O devices. While it exposes sensor data and configuration options, it doesn't provide direct access to the device registers. The IIO subsystem abstracts the low-level hardware details and exposes a more generic interface for reading sensor data and configuring sensor parameters.

- /dev/input/devices/: This directory contains device files representing input devices, such as keyboards, mice, touchscreens, and some sensors (including accelerometers). However, these device files are designed to report input events (e.g., key presses, mouse movements, touch events) or sensor data in a pre-defined format, rather than providing direct access to the device registers.

To read and write directly to the accelerometer registers, you typically need to use a lower-level interface that interacts with the device's communication protocol (e.g., I2C, SPI). This is often done through a character device file (/dev/i2c-* or /dev/spidev*) or a custom kernel driver that exposes a device file specifically for the accelerometer.

In the example provided earlier, the code assumes the existence of a device file /dev/adxl345 that represents the ADXL345 accelerometer. This device file could be created by a custom kernel driver or a generic driver that supports register-level access to the accelerometer over the I2C or SPI interface.

Using the IIO or input device files is generally more convenient when you only need to read the sensor data in a standardized format. However, if you need to perform low-level operations, such as configuring the accelerometer's registers, calibrating the device, or accessing advanced features, you'll need to use a lower-level interface that provides direct register access.

It's important to note that the availability of such low-level interfaces and the specific device files may vary depending on the kernel version, hardware platform, and the drivers installed on your system. Consulting the accelerometer's documentation and the kernel driver documentation is recommended to understand the available interfaces and the appropriate way to interact with the device on your specific system.

User-Space Application (adxl_app.c)

#include <stdio.h>

#include <stdlib.h>

#include <unistd.h>

#include <pthread.h>

#include <fcntl.h>

#include <sys/ioctl.h>

#define ADXL_DEV "/dev/input/event0"

static int adxl_fd;

static pthread_mutex_t mutex = PTHREAD_MUTEX_INITIALIZER;

static pthread_cond_t cond = PTHREAD_COND_INITIALIZER;

static int16_t accel_data[3];

void *accel_reader_thread(void *arg) {

while (1) {

pthread_mutex_lock(&mutex);

pthread_cond_wait(&cond, &mutex);

read(adxl_fd, accel_data, sizeof(accel_data));

printf("Acceleration: X=%d, Y=%d, Z=%d\n", accel_data[0], accel_data[1], accel_data[2]);

pthread_mutex_unlock(&mutex);

}

return NULL;

}

int main() {

pthread_t thread;

adxl_fd = open(ADXL_DEV, O_RDONLY);

if (adxl_fd < 0) {

perror("Failed to open ADXL device");

exit(EXIT_FAILURE);

}

pthread_create(&thread, NULL, accel_reader_thread, NULL);

while (1) {

/* Other application logic */

sleep(1);

}

close(adxl_fd);

return 0;

}

Kernel Module (adxl_driver.c)

#include <linux/module.h>

#include <linux/kernel.h>

#include <linux/interrupt.h>

#include <linux/workqueue.h>

#include <linux/semaphore.h>

/* ADXL device and driver structures */

struct adxl_dev {

struct semaphore sem;

int16_t accel_data[3]; /* x, y, z acceleration data */

/* Other ADXL device-specific members */

};

static struct adxl_dev *adxl_dev;

/* Top-half interrupt handler */

irqreturn_t adxl_interrupt_handler(int irq, void *dev_id) {

if (!down_trylock(&adxl_dev->sem))

queue_work(adxl_wq, &adxl_work);

else

up(&adxl_dev->sem);

return IRQ_HANDLED;

}

/* Bottom-half work queue handler */

static void adxl_work_handler(struct work_struct *work) {

/* Read acceleration data from ADXL device */

read_accel_data(adxl_dev->accel_data);

up(&adxl_dev->sem);

}

static DECLARE_WORK(adxl_work, adxl_work_handler);

static struct workqueue_struct *adxl_wq;

/* Character device interface for user-space application */

static ssize_t adxl_read(struct file *filp, char __user *buf, size_t count, loff_t *f_pos) {

if (down_interruptible(&adxl_dev->sem))

return -ERESTARTSYS;

if (copy_to_user(buf, adxl_dev->accel_data, sizeof(adxl_dev->accel_data)))

return -EFAULT;

up(&adxl_dev->sem);

return sizeof(adxl_dev->accel_data);

}

static const struct file_operations adxl_fops = {

.read = adxl_read,

/* Other file operations */

};

static int __init adxl_init(void) {

adxl_wq = create_singlethread_workqueue("adxl_wq");

adxl_dev = kzalloc(sizeof(*adxl_dev), GFP_KERNEL);

sema_init(&adxl_dev->sem, 1);

/* Register ADXL device, request IRQ, and set up interrupt handler */

return 0;

}

static void __exit adxl_exit(void) {

destroy_workqueue(adxl_wq);

kfree(adxl_dev);

}

module_init(adxl_init);

module_exit(adxl_exit);

EEPROM (specifically AT24C02)

dd if=/dev/zero of=/dev/eeprom bs=1(32) count=<number_of_bytes_to_clear> //8

dd if=input_file.bin of=/dev/eeprom bs=1 count=128

dd if=/dev/eeprom of=output_file.bin bs=1 count=128

/dev/eeprom -- >  /sys/bus/nvmem

#include <stdio.h>

#include <stdlib.h>

#include <fcntl.h>

#include <unistd.h>

#include <sys/ioctl.h>

#include <linux/i2c-dev.h>

#define EEPROM_DEVICE "/dev/nvram0"  // Adjust device name according to your system

#define EEPROM_SIZE 256  // Size of AT24C02 EEPROM in bytes

int main() {

int eeprom_fd;

// Open the EEPROM device

eeprom_fd = open(EEPROM_DEVICE, O_RDWR);

if (eeprom_fd < 0) {

perror("Failed to open EEPROM device");

return EXIT_FAILURE;

}

// Seek to the beginning of the EEPROM

lseek(eeprom_fd, 0, SEEK_SET);

// Write data to EEPROM

char write_data[] = "Hello, EEPROM!";

if (write(eeprom_fd, write_data, sizeof(write_data)) < 0) {

perror("Failed to write to EEPROM");

close(eeprom_fd);

return EXIT_FAILURE;

}

// Seek back to the beginning of the EEPROM

lseek(eeprom_fd, 0, SEEK_SET);

// Read data from EEPROM

char read_data[EEPROM_SIZE];

if (read(eeprom_fd, read_data, sizeof(read_data)) < 0) {

perror("Failed to read from EEPROM");

close(eeprom_fd);

return EXIT_FAILURE;

}

// Print the read data

printf("Data read from EEPROM: %s\n", read_data);

// Close the EEPROM device

close(eeprom_fd);

return EXIT_SUCCESS;

}

----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

#include <stdio.h>

#include <stdlib.h>

#include <unistd.h>

#include <pthread.h>

#include <fcntl.h>

#include <sys/types.h>

#include <sys/stat.h>

/* Define the device file paths */

#define ADC_DEVICE "/dev/adc"

#define RTC_DEVICE "/dev/rtc"

#define EEPROM_DEVICE "/dev/eeprom"

/* Define the delay or sleep time in microseconds */

#define DELAY_TIME 100000  /* 100 milliseconds */

/* Define the shared data structure */

struct shared_data {

int adc_value;

unsigned long rtc_timestamp;

pthread_mutex_t mutex;

pthread_cond_t cond;

};

/* Declare the thread functions */

void *adc_thread_fn(void *data);

void *rtc_thread_fn(void *data);

void *eeprom_thread_fn(void *data);

/* Shared data object */

struct shared_data shared_data;

int main()

{

pthread_t adc_thread, rtc_thread, eeprom_thread;

/* Initialize the mutex and condition variable */

pthread_mutex_init(&shared_data.mutex, NULL);

pthread_cond_init(&shared_data.cond, NULL);

/* Create the ADC thread */

pthread_create(&adc_thread, NULL, adc_thread_fn, NULL);

/* Create the RTC thread */

pthread_create(&rtc_thread, NULL, rtc_thread_fn, NULL);

/* Create the EEPROM thread */

pthread_create(&eeprom_thread, NULL, eeprom_thread_fn, NULL);

/* Wait for the threads to finish */

pthread_join(adc_thread, NULL);

pthread_join(rtc_thread, NULL);

pthread_join(eeprom_thread, NULL);

/* Destroy the mutex and condition variable */

pthread_mutex_destroy(&shared_data.mutex);

pthread_cond_destroy(&shared_data.cond);

return 0;

}

void *adc_thread_fn(void *data)

{

int adc_fd;

/* Open the ADC device file */

adc_fd = open(ADC_DEVICE, O_RDONLY);

if (adc_fd == -1) {

perror("Failed to open ADC device");

return NULL;

}

while (1) {

int adc_value;

/* Read the ADC value */

adc_value = /* ... read from ADC device ... */

/* Acquire the mutex */

pthread_mutex_lock(&shared_data.mutex);

/* Update the shared data */

shared_data.adc_value = adc_value;

/* Signal the condition variable */

pthread_cond_signal(&shared_data.cond);

/* Release the mutex */

pthread_mutex_unlock(&shared_data.mutex);

/* Sleep for the specified delay time */

usleep(DELAY_TIME);

}

/* Close the ADC device file */

close(adc_fd);

return NULL;

}

void *rtc_thread_fn(void *data)

{

int rtc_fd;

/* Open the RTC device file */

rtc_fd = open(RTC_DEVICE, O_RDONLY);

if (rtc_fd == -1) {

perror("Failed to open RTC device");

return NULL;

}

while (1) {

unsigned long rtc_timestamp;

/* Read the RTC timestamp */

rtc_timestamp = /* ... read from RTC device ... */

/* Acquire the mutex */

pthread_mutex_lock(&shared_data.mutex);

/* Update the shared data */

shared_data.rtc_timestamp = rtc_timestamp;

/* Signal the condition variable */

pthread_cond_signal(&shared_data.cond);

/* Release the mutex */

pthread_mutex_unlock(&shared_data.mutex);

/* Sleep for the specified delay time */

usleep(DELAY_TIME);

}

/* Close the RTC device file */

close(rtc_fd);

return NULL;

}

void *eeprom_thread_fn(void *data)

{

int eeprom_fd;

/* Open the EEPROM device file */

eeprom_fd = open(EEPROM_DEVICE, O_WRONLY);

if (eeprom_fd == -1) {

perror("Failed to open EEPROM device");

return NULL;

}

while (1) {

int adc_value;

unsigned long rtc_timestamp;

/* Acquire the mutex */

pthread_mutex_lock(&shared_data.mutex);

/* Wait for the condition variable to be signaled */

pthread_cond_wait(&shared_data.cond, &shared_data.mutex);

/* Copy the shared data */

adc_value = shared_data.adc_value;

rtc_timestamp = shared_data.rtc_timestamp;

/* Release the mutex */

pthread_mutex_unlock(&shared_data.mutex);

/* Write the data to the EEPROM device */

/* ... write adc_value and rtc_timestamp to EEPROM device ... */

}

/* Close the EEPROM device file */

close(eeprom_fd);

return NULL;

}

#include <stdio.h>

#include <stdlib.h>

#include <stdint.h>

#include <fcntl.h>

#include <unistd.h>

#include <linux/i2c-dev.h>

#include <sys/ioctl.h>

#include <time.h>

// Define the EEPROM I2C address and page size

#define EEPROM_ADDR 0x50

#define PAGE_SIZE 16

// Function to get current timestamp

void get_timestamp(char *buffer, size_t size) {

time_t now = time(NULL);

struct tm *t = localtime(&now);

strftime(buffer, size, "%Y-%m-%d %H:%M:%S", t);

}

// Function to write a page of data to EEPROM

int eeprom_write_page(int fd, uint16_t page, uint8_t *data, size_t length) {

if (length > PAGE_SIZE) {

fprintf(stderr, "Error: Data length exceeds page size\n");

return -1;

}

uint8_t buffer[PAGE_SIZE + 2];

buffer[0] = (page >> 8) & 0xFF; // High byte of page address

buffer[1] = page & 0xFF;        // Low byte of page address

for (size_t i = 0; i < length; i++) {

buffer[2 + i] = data[i];

}

if (write(fd, buffer, length + 2) != length + 2) {

perror("Write failed");

return -1;

}

usleep(5000); // Wait for the write cycle to complete

return 0;

}

// Function to read a page of data from EEPROM

int eeprom_read_page(int fd, uint16_t page, uint8_t *data, size_t length) {

if (length > PAGE_SIZE) {

fprintf(stderr, "Error: Data length exceeds page size\n");

return -1;

}

uint8_t buffer[2];

buffer[0] = (page >> 8) & 0xFF; // High byte of page address

buffer[1] = page & 0xFF;        // Low byte of page address

if (write(fd, buffer, 2) != 2) {

perror("Write failed");

return -1;

}

if (read(fd, data, length) != length) {

perror("Read failed");

return -1;

}

return 0;

}

int main() {

int fd;

const char *i2c_dev = "/dev/i2c-1"; // Change as needed

// Open the I2C device

if ((fd = open(i2c_dev, O_RDWR)) < 0) {

perror("Failed to open the i2c bus");

exit(1);

}

// Specify the EEPROM device address

if (ioctl(fd, I2C_SLAVE, EEPROM_ADDR) < 0) {

perror("Failed to acquire bus access and/or talk to slave");

close(fd);

exit(1);

}

// Prepare data to write

uint8_t data_to_write[PAGE_SIZE];

for (int i = 0; i < 6; i++) {

data_to_write[i] = i + 1; // Example temperature and ADXL data

}

char timestamp[20];

get_timestamp(timestamp, sizeof(timestamp));

snprintf((char *)data_to_write + 6, PAGE_SIZE - 6, "%s", timestamp);

// Write the data to page 0

if (eeprom_write_page(fd, 0, data_to_write, PAGE_SIZE) != 0) {

fprintf(stderr, "Error writing data to EEPROM\n");

close(fd);

exit(1);

}

// Read the data back

uint8_t data_read[PAGE_SIZE];

if (eeprom_read_page(fd, 0, data_read, PAGE_SIZE) != 0) {

fprintf(stderr, "Error reading data from EEPROM\n");

close(fd);

exit(1);

}

// Print the data read

printf("Data read from EEPROM:\n");

for (int i = 0; i < 6; i++) {

printf("Byte %d: %d\n", i, data_read[i]);

}

printf("Timestamp: %s\n", data_read + 6);

// Close the I2C device

close(fd);

return 0;

}

----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Boot Time Optimization:

Pass the command line argument while booting the kernel in uboot stage in embedded devices.

In system(x-86) set the arguments in grub menu or file (/etc/default/grub).

Set the boot argument is “init=/lib/systemd/systemd-bootchart”.

This will provide booting time with analysis of system while boot the running process.

Load /boot/vmlinuz-4

Load /boot/dts/

Set env boot args

Bootz {addrs}

------------------------------------------------------------------------------------------------------------------------------

setenv bootargs rootfstype=${mmcrootfstype} root=/dev/mmcblk0p1 console=ttyS0,115200n8 rodata=off nokaslr kgdb=ttyS0,115200 kgdboc=ttyS0,115200n8 kgdbwait rootwait;

4.19.94Rohith-kernel-customization-g0bb6869c7-dirty

km-bbb-am335x.dtb

load mmc 1:1 ${loadaddr} /boot/vmlinuz-4.19.94Rohith-kernel-customization-g0bb6869c7-dirty

load mmc 1:1 ${fdtaddr} /boot/dtbs/4.19.94Rohith-kernel-customization-g0bb6869c7-dirty/km-bbb-am335x.dtb

setenv bootargs console=${console} root=/dev/mmcblk1p1 rootfstype=${mmcrootfstype} init=/lib/systemd/systemd-bootchart

bootz ${loadaddr} - ${fdtaddr}

setenv bootargs console=${console} root=/dev/mmcblk1p1 rootfstype=${mmcrootfstype} crashkernel=64M maxcpus=1

From target to host

km@KernelMasters:~$ scp km@192.168.43.213:~/bootchart-20240513-1353.svg .

This command prints an SVG graphic detailing which system service have been started at what time, highlighting the time they spent on initialization.

$ systemd-analyze plot >bootup.svg

$ inkscape bootup.svg

systemctl status kdump-tools.service

systemctl status kexec.service

km@KM-BBB:~$ cat /etc/kdump.conf

path /var/crash

core_collector makedumpfile -l --message-level 1 -d 31

km@KM-BBB:~$ cat /usr/sbin/kexec.sh

#!/bin/sh

ARGS="console=ttyO0,115200n8 root=/dev/mmcblk1p1 rootfstype=ext4 rootwait"

kexec -d \

--type zImage -p /boot/vmlinuz-4.19.94Rohith-kernel-customization-g0bb6869c7-dirty \

--command-line="$ARGS maxcpus=1 reset_devices"

# CONFIG_SYSCON_REBOOT_MODE is not set

# CONFIG_PANIC_ON_OOPS is not set

CONFIG_PANIC_ON_OOPS_VALUE=0

CONFIG_PANIC_TIMEOUT=0

$/etc/default/kdump-tools

USE_KDUMP=1

KDUMP_KERNEL="/var/lib/kdump/initrd.img-4.19.94Rohith-kernel-customization-g0bb6869c7-dirty"

KDUMP_INITRD="/var/lib/kdump/initrd.img"

KDUMP_COMMANDLINE="console=ttyO0,115200n8 root=/dev/mmcblk1p1 rootfstype=ext4 rootwait maxcpus=1 reset_devices"

/etc/systemd/system/kdump.service

Real-time Data Logger Application

#define _GNU_SOURCE

#include <stdio.h>

#include <stdlib.h>

#include <fcntl.h>

#include <unistd.h>

#include <pthread.h>

#include <string.h>

#include <errno.h>

#include <sys/epoll.h>

#include <sys/timerfd.h>

#include <time.h>

#include <sched.h>

#include <linux/i2c-dev.h>

#include <sys/ioctl.h>

#define MAX_EVENTS 10

#define BUTTON_GPIO "/sys/class/gpio/gpio60/value"

#define I2C_DEVICE "/dev/i2c-1"

#define ADXL345_ADDR 0x53

#define EEPROM_ADDR 0x50

#define RTC_ADDR 0x68

// Function prototypes

void *button_thread(void *arg);

void *sensor_thread(void *arg);

int read_accelerometer(int fd, int16_t *x, int16_t *y, int16_t *z);

int read_temperature(int fd, float *temp);

int read_rtc(int fd, struct tm *time);

int write_to_eeprom(int fd, uint16_t address, uint8_t *data, size_t len);

// Global variables

volatile int g_button_pressed = 0;

int g_i2c_fd;

int main() {

pthread_t button_tid, sensor_tid;

pthread_attr_t attr;

struct sched_param param;

// Open I2C device

g_i2c_fd = open(I2C_DEVICE, O_RDWR);

if (g_i2c_fd < 0) {

perror("Failed to open I2C device");

return 1;

}

// Set up real-time priority for threads

pthread_attr_init(&attr);

pthread_attr_setinheritsched(&attr, PTHREAD_EXPLICIT_SCHED);

pthread_attr_setschedpolicy(&attr, SCHED_FIFO);

param.sched_priority = 80;

pthread_attr_setschedparam(&attr, &param);

// Create button thread

if (pthread_create(&button_tid, &attr, button_thread, NULL) != 0) {

perror("Failed to create button thread");

return 1;

}

// Create sensor thread

if (pthread_create(&sensor_tid, &attr, sensor_thread, NULL) != 0) {

perror("Failed to create sensor thread");

return 1;

}

// Wait for threads to finish (they won't in this example)

pthread_join(button_tid, NULL);

pthread_join(sensor_tid, NULL);

close(g_i2c_fd);

return 0;

}

void *button_thread(void *arg) {

int fd, epfd;

struct epoll_event ev, events[MAX_EVENTS];

char buf[2];

// Open GPIO value file

fd = open(BUTTON_GPIO, O_RDONLY);

if (fd < 0) {

perror("Failed to open GPIO file");

return NULL;

}

// Create epoll instance

epfd = epoll_create1(0);

if (epfd < 0) {

perror("Failed to create epoll instance");

close(fd);

return NULL;

}

// Add GPIO file to epoll

ev.events = EPOLLPRI | EPOLLET;

ev.data.fd = fd;

if (epoll_ctl(epfd, EPOLL_CTL_ADD, fd, &ev) < 0) {

perror("Failed to add GPIO file to epoll");

close(fd);

close(epfd);

return NULL;

}

// Main loop

while (1) {

int nfds = epoll_wait(epfd, events, MAX_EVENTS, -1);

if (nfds < 0) {

perror("epoll_wait failed");

break;

}

for (int n = 0; n < nfds; ++n) {

if (events[n].data.fd == fd) {

lseek(fd, 0, SEEK_SET);

if (read(fd, buf, sizeof(buf)) > 0) {

if (buf[0] == '1') {

g_button_pressed = 1;

}

}

}

}

}

close(fd);

close(epfd);

return NULL;

}

void *sensor_thread(void *arg) {

int16_t x, y, z;

float temp;

struct tm rtc_time;

uint8_t eeprom_data[16];

uint16_t eeprom_address = 0;

while (1) {

if (g_button_pressed) {

// Read accelerometer

if (read_accelerometer(g_i2c_fd, &x, &y, &z) < 0) {

fprintf(stderr, "Failed to read accelerometer\n");

}

// Read temperature

if (read_temperature(g_i2c_fd, &temp) < 0) {

fprintf(stderr, "Failed to read temperature\n");

}

// Read RTC

if (read_rtc(g_i2c_fd, &rtc_time) < 0) {

fprintf(stderr, "Failed to read RTC\n");

}

// Prepare data for EEPROM

memcpy(eeprom_data, &x, sizeof(x));

memcpy(eeprom_data + 2, &y, sizeof(y));

memcpy(eeprom_data + 4, &z, sizeof(z));

memcpy(eeprom_data + 6, &temp, sizeof(temp));

memcpy(eeprom_data + 10, &rtc_time, sizeof(rtc_time));

// Write to EEPROM

if (write_to_eeprom(g_i2c_fd, eeprom_address, eeprom_data, sizeof(eeprom_data)) < 0) {

fprintf(stderr, "Failed to write to EEPROM\n");

}

eeprom_address += sizeof(eeprom_data);

g_button_pressed = 0;

printf("Data logged: Accel(%d,%d,%d) Temp(%.2f) Time(%02d:%02d:%02d)\n",

x, y, z, temp, rtc_time.tm_hour, rtc_time.tm_min, rtc_time.tm_sec);

}

// Sleep for a short time to reduce CPU usage

usleep(10000);  // 10ms

}

return NULL;

}

int read_accelerometer(int fd, int16_t *x, int16_t *y, int16_t *z) {

uint8_t reg = 0x32;  // X-axis data register

uint8_t data[6];

if (ioctl(fd, I2C_SLAVE, ADXL345_ADDR) < 0) {

return -1;

}

if (write(fd, &reg, 1) != 1) {

return -1;

}

if (read(fd, data, 6) != 6) {

return -1;

}

*x = (data[1] << 8) | data[0];

*y = (data[3] << 8) | data[2];

*z = (data[5] << 8) | data[4];

return 0;

}

int read_temperature(int fd, float *temp) {

// Implement temperature sensor reading here

// This is a placeholder implementation

*temp = 25.5;

return 0;

}

int read_rtc(int fd, struct tm *time) {

// Implement RTC reading here

// This is a placeholder implementation

time_t t = time(NULL);

struct tm *local_time = localtime(&t);

memcpy(time, local_time, sizeof(struct tm));

return 0;

}

int write_to_eeprom(int fd, uint16_t address, uint8_t *data, size_t len) {

uint8_t buf[len + 2];

buf[0] = (address >> 8) & 0xFF;

buf[1] = address & 0xFF;

memcpy(buf + 2, data, len);

if (ioctl(fd, I2C_SLAVE, EEPROM_ADDR) < 0) {

return -1;

}

if (write(fd, buf, len + 2) != len + 2) {

return -1;

}

// Wait for write to complete

usleep(5000);  // 5ms

return 0;

}

ADXL345:

#define DEVICE_NAME "adxl_interrupt"

#define GPIO_NUM 17

#define I2C_BUS_NUM 1

#define ADXL345_ADDR 0x53

#define ADXL345_REG_DATA_FORMAT 0x31

#define ADXL345_REG_POWER_CTL 0x2D

#define ADXL345_REG_DATAX0 0x32

static int irq_number;

static int major;

static char data[6]; // Buffer to store x, y, z axis data

static DECLARE_WAIT_QUEUE_HEAD(wq);

static int flag = 0;

static struct i2c_adapter *adap;

static struct i2c_client *client;

static irqreturn_t adxl_isr(int irq, void *dev_id)

{

struct i2c_msg msgs[2];

u8 reg = ADXL345_REG_DATAX0;

msgs[0].addr = ADXL345_ADDR;

msgs[0].flags = 0;

msgs[0].len = 1;

msgs[0].buf = &reg;

msgs[1].addr = ADXL345_ADDR;

msgs[1].flags = I2C_M_RD;

msgs[1].len = 6;

msgs[1].buf = data;

i2c_transfer(adap, msgs, 2);

flag = 1;

wake_up_interruptible(&wq);

return IRQ_HANDLED;

}

static ssize_t device_read(struct file *file, char __user *buffer, size_t len, loff_t *offset)

{

wait_event_interruptible(wq, flag != 0);

flag = 0;

if (copy_to_user(buffer, data, 6))

return -EFAULT;

return 6;

}

static struct file_operations fops = {

.owner = THIS_MODULE,

.read = device_read,

};

static int __init adxl_init(void)

{

int result;

struct i2c_board_info info = {

I2C_BOARD_INFO("adxl345", ADXL345_ADDR)

};

adap = i2c_get_adapter(I2C_BUS_NUM);

client = i2c_new_device(adap, &info);

if (!client) {

printk(KERN_ALERT "Failed to create i2c client\n");

return -ENODEV;

}

i2c_smbus_write_byte_data(client, ADXL345_REG_DATA_FORMAT, 0x01); // Set range to +/- 4g

i2c_smbus_write_byte_data(client, ADXL345_REG_POWER_CTL, 0x08);   // Start measuring

major = register_chrdev(0, DEVICE_NAME, &fops);

if (major < 0) {

printk(KERN_ALERT "Registering char device failed with %d\n", major);

return major;

}

if (!gpio_is_valid(GPIO_NUM)) {

printk(KERN_ALERT "Invalid GPIO\n");

unregister_chrdev(major, DEVICE_NAME);

return -ENODEV;

}

gpio_request(GPIO_NUM, "sysfs");

gpio_direction_input(GPIO_NUM);

gpio_set_debounce(GPIO_NUM, 200);

gpio_export(GPIO_NUM, false);

irq_number = gpio_to_irq(GPIO_NUM);

result = request_irq(irq_number, adxl_isr, IRQF_TRIGGER_RISING, DEVICE_NAME, NULL);

if (result) {

printk(KERN_ALERT "GPIO IRQ request failed\n");

gpio_unexport(GPIO_NUM);

gpio_free(GPIO_NUM);

unregister_chrdev(major, DEVICE_NAME);

return result;

}

printk(KERN_INFO "ADXL module loaded\n");

return 0;

}

static void __exit adxl_exit(void)

{

free_irq(irq_number, NULL);

gpio_unexport(GPIO_NUM);

gpio_free(GPIO_NUM);

unregister_chrdev(major, DEVICE_NAME);

i2c_unregister_device(client);

printk(KERN_INFO "ADXL module unloaded\n");

}

module_init(adxl_init);

module_exit(adxl_exit);

MODULE_LICENSE("GPL");

MODULE_AUTHOR("Author");

MODULE_DESCRIPTION("A simple ADXL interrupt module");